Master Specification    04 05 31        TotalFlash®

Date: 07/25/2018

This Section may be used as a narrow scope, stand-alone Section or may be modified and incorporated into broad scope Section 04 20 00 Unit Masonry

SECTION 04

Cavity Wall Flashing/Drainage System

PART 1 - General

  - SUMMARY

- Product guide specification can be used to specify TotalFlash® which is an all-inclusive unitized flashing/drainage system. Product specification typically inserted into Section 04800 – MASONRY ASSEMBLIES or other similar masonry specification sections listed below. Section includes Flashing, Cavity Wall Drainage, Drip Edge, Termination Bar and Drainage Weeps. Replaces the requirement for flashing, weeps, mortar collection product, drip edge and termination bar.

Edit following sections as determined by specification writer:

- Related Sections:

1. 04 05 23   Masonry Accessories

2. 04 21 13   Brick Masonry

3. 04 22 00   Concrete Unit Masonry

4. 04 22 23   Architectural Concrete Unit Masonry

5. 04 42 00   Exterior Stone Cladding

6. 04 72 00   Cast Stone Masonry

7. 07 60 00   Flashing and Sheet Metal

8. 07 65 00   Flexible Flashing

- Product replaces requirements for Flashing, Mortar Collection System, WeepVent, Drip Edge and Termination bar as required in Sections 04 and 07.

  - REFERENCES

- Industry Standards:

  - American Society for Testing Materials (ASTM)

  - Brick Industry Association (BIA)

  - Mason Contractors Association of American (MCAA)

- Industry Standards:

  - BIA Tech Note: Brick Construction - #7

Water Penetration Resistance - Design and Detail

  - BIA Tech Note: Brick Construction - #28B

Brick Veneer/Steel Stud Walls

  - DEFINITIONS

- Terms:

  - Cavity Wall Flashing

  - Foundation Sill Flashing

  - Through Wall Flashing

  - Termination Bar

  - Base Flashing

  - Shelf Angle Flashing

  - SUBMITTALS

- Provide in accordance with Section 01330 (Submittal Procedures)

  - Product data and installation instructions.

  - TotalFlash® can be made with all flashing membranes.TotalFlash® is factory produced in a controlled environment vs. field assembly. Two sample sections demonstrating lap joint available on request.

PART 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Mortar Net Solutions

6575 Daniel Burnham Drive, Suite G, Portage, IN  46368

Ph: 800-664-6638 	 Fax:  219-787-5088         www. MortarNet.com

  - PRODUCTS

Mortar Net’s “TotalFlash®” system is designed to replace traditional flashing components. Custom

cut to size configurations for headers and door openings are available. The TotalFlash® Cavity Wall Drainage system, basis of design is 45-mil membrane of EPDM rubber (ethylene propylene diene monomer rubber; recycled polyester 1/4-inch x 10-inch Mortar Net Solutions drainage mat; and 28-gauge 304 stainless steel drip edge; high strength corrosion and UV resistant plastic termination bar; and integral no-clog weep tabs. Self-tapping grommeted hex head screws are #14 x 2-inch for metal or wood stud, cast in place concrete or concrete masonry units.

Other membranes are available: Thermoplastic with UV stabilizers and non-migratory plasticizers,

40-mil Rubberized Asphalt, 40-mil Thermoplastic Olefin (TPO) or 5-ounce Copper Laminate, 24-gauge, Stainless Steel laminated fabric membrane or designer’s choice.

Seamless factory molded 14-inch inside and outside corner boots, 4-inch and 6-inch end dams are produced from synthetic rubber.

2.03 PANEL SIZES

| New Construction Standard Size | 18-inch x 5-1/2 foot |
| --- | --- |

| Additional Sizes | 12-inch x 5-1/2 foot 24-inch x 5-1/2 foot |
| --- | --- |
| CUSTOM SIZES & CONFIGURATIONS AVAILABLE FOR MASONRY OPENINGS AND SPECIAL CONDIDITIONS |  |

- Mortar Collection Drainage Mat and Drainage Weep Tabs:

Recycled polyester material impregnated with biocide to resist mold and flame retardant. High Loft Non- Woven mesh designed to allow moisture to migrate to the integrated weep tabs; product adhered to the flashing membrane.

  - Thickness: 1/4 inch (6.35 mm)

  - Height: 10 inches

  - Length: 5 feet

- Drip Edge:

304 Stainless Steel Drip Edge pre-attached to the flashing membrane and designed to divert moisture away from the masonry wall.

- 28-gauge 304 Stainless Steel with hemmed drip edge

- Length: 5.0 feet

- Width: 3.0 inches Other Products Available; Cold rolled copper or 24-gauge Kynar finish galvanized steel; colors include Almond, Terra-Cotta, Gray and Tan (all options available separately in 10-foot lengths).

- Termination Bar:

Pre-attached termination bar is designed to fasten flashing system to the substrate.

  - Strip manufactured from high strength corrosion resistance plastic with pre-drilled holes for attachment.

  - Length 5 feet

  - Hole spacing 6 inches

  - Other Products Available; 16-gauge stainless steel termination bar flat or with sealant bend.

- Screws:

Provided self-tapping hex head screws designed to allow attachment to Masonry, Concrete, Wood or Steel Stud.

- #14 x 2-inch

- Quantity per box 100 Self-tapping #14 x 2-inch hex screws with self-sealing gaskets.

PART 3 – Execution

  - INSTALLATION

  - Install Flashing/Drainage System in accordance with manufacturer’s installation instructions. Instructions can be found at:

https://mortarnet.com/totalflash-panel/totalflash-installation/

END OF SECTION